Installation 
1. NOTE: If a new intake manifold or IMRC housing assembly is being installed, transfer all necessary components onto new intake manifold or IMRC housing assembly using new gaskets. 

Clean and inspect all sealing surfaces. Upper sealing surfaces or IMRC housing assemblies and intake manifold mating surfaces should be cleaned with a suitable solvent to remove adhesive residue. Inspect intake manifold mating surfaces and upper IMRC assembly surfaces to ensure that no load limiting spacers (washers) are stuck to the sealing surfaces. The load limiting spacers are part of the intake manifold upper gasket. New load limiter spacers will be included with the replacement intake manifold upper gasket as an assembly. 

2. Remove protective housing paper from adhesive backing on intake manifold upper gasket and install onto top of RH IMRC assembly using tapered pins at end bolt hole locations for proper gasket alignment. Repeat procedure for LH assembly. 

3. Assemble IMRC assemblies to intake manifold and install four IMRC retaining bolts. Finger-tighten bolts. 

4. Install fuel injection supply manifold onto fuel injectors and install four retaining bolts. Tighten to 8-12 Nm (71-106 lb-in). 

5. Position new intake manifold gaskets onto cylinder heads using gaskets' integral locating pins to align gaskets. 

6. Place intake manifold under engine control sensor wiring and connect fuel charging wiring (9D930) to idle air control valve and throttle position sensor or throttle position sensors (depending upon application). 

7. Move intake manifold into position and install new EGR valve gasket and two EGR valve retaining bolts. Finger-tighten bolts. 

8. Install longer bolts and stud bolts in outer bolt holes of intake manifold. Do not tighten. 

9. Install shorter bolts and stud bolts in inner bolt holes of intake manifold. Hand-tighten all 20 fasteners. 

10. Following sequence shown, tighten fasteners 1 through 20 in numerical sequence as follows: 

· Fasteners 5, 7, 9 and 11 to 12-15 Nm (9-11 lb-ft). 

· All other fasteners to 18-22 Nm (14-16 lb-ft). 

· Then tighten all fasteners in numerical sequence by rotating an additional 85-95 degrees. 

10. Intake Manifold Torque Sequence 



11. Tighten four intake manifold to IMRC housing retaining bolts (A-D) to 8-10 Nm (71-88 lb-in). Tighten fasteners A through D in sequence shown by rotating 85-95 degrees. 

IMRC Torque Sequence 


[image: image1.png]@-swo
h-sor

FRONT OF ENGINE

A23816-A





